## Technical Summary — Post-Import Integration Actions

After importing the AHMI Dynamic Route ETA Billboard System asset into the main Unity project, the following integration actions were performed.

## 1. Main Scene Structure Integration

The imported ETA system was placed in the main scene as:

AHMI_DynamicRouteSystem

This object now acts as the central entry point of the imported asset and contains the ETA-related bridge and traffic delay logic.

Configured components:

AHMI_DynamicRouteSystem
├── DynamicRouteBridge
├── ParkingTrafficProvider
└── DynamicRouteDebugTester   // optional, disabled for real integration

## 2. Real Vehicle Connection

The ETA system was connected to the actual vehicle used by the main simulation:

NPC Cars
└── Car 5

The following component was added to Car 5:

CarETAOrchestrator

The previous generic placeholder object:

Car_Autonomous

was not used for the real wiring, because Car 5 is the actual vehicle driven by the project’s existing navigation system.

## 3. Car ETA Orchestrator Configuration

CarETAOrchestrator was configured on Car 5.

Main parameters:

Speed Meters Per Second: 3
Waypoint Reach Threshold: 1
Default Pedestrian Crossing Delay Seconds: 8
Safety Buffer Seconds: 5
Internal Calculation Interval: 1
Billboard Publish Interval: 10
ETA Change Threshold Seconds: 15
Round ETA To Nearest Seconds: 30
Log Debug Info: false

The delay provider was connected as:

Parking Traffic Provider → AHMI_DynamicRouteSystem

This allows CarETAOrchestrator to include traffic-related delay in the ETA computation.

## 4. Dynamic Route Bridge Wiring

DynamicRouteBridge on AHMI_DynamicRouteSystem was connected to the real ETA orchestrator:

DynamicRouteBridge
└── Car ETA Orchestrator → Car 5

This enables the bridge to pass externally generated route data to the ETA system.

## 5. DynamicRouteBridge Script Adaptation

DynamicRouteBridge.cs was extended to support the main project’s native route format:

C#List<TrafficNode>

A new method was added:

C#ReceiveTrafficNodeRoute(List<TrafficNode>trafficNodes, stringpickupLabel)

This converts the project’s TrafficNode path into a Transform route compatible with CarETAOrchestrator.

The bridge now supports route inputs from:

List<Transform>
Transform[]
List<GameObject>
GameObject[]
List<RouteEndpoint>
RouteEndpoint[]
List<TrafficNode>
TrafficNode[]

This preserves the asset’s modularity and compatibility with different project versions.

## 6. Main Routing System Analysis

The main project already contains a complete route network system:

TrafficNetwork
TrafficNode

The TrafficNetwork script provides:

C#GetClosestNode(Vector3position)
GetPath(TrafficNodestart, TrafficNodeend)

The route graph is based on TrafficNode.nextNodes, which enforces legal one-way driving connections.

Therefore, no custom pathfinding system was added to the ETA asset.
The imported ETA system reuses the main project’s existing routing logic.

## 7. ValetGuidanceSystem Integration

ValetGuidanceSystem.cs was modified to connect the existing valet logic to the ETA billboard system.

A new integration section was added:

C#[Header("AHMI ETA Billboard Integration")]
[SerializeField] privateDynamicRouteBridgedynamicRouteBridge;
[SerializeField] privateParkingTrafficProviderparkingTrafficProvider;
[SerializeField] privateboolenableEtaBillboardIntegration=true;

This allows CentralSystem to communicate with the imported AHMI ETA asset.

## 8. Pickup Route Transmission

The ETA update was connected to the exact point where the main system recalls a parked vehicle.

The integration point is the transition:

Parked → MovingToPickUp

Inside this state transition, the main system already:

1. selects a free pickup spot
2. calculates a TrafficNode path
3. sends the path to SmartCarNavigator

The same calculated path is now also sent to the ETA system through:

C#dynamicRouteBridge.ReceiveTrafficNodeRoute(path, pickupLabel);

This ensures the billboard ETA uses the same route that the vehicle physically follows.

## 9. Pickup Label Mapping

A helper method was added to convert pickup spot index into a readable label:

pickUpSpots[0] → A
pickUpSpots[1] → B
pickUpSpots[2] → C

This allows the billboard to display:

Pickup A
Pickup B
Pickup C

depending on the pickup area assigned by the valet system.

## 10. Traffic Delay Data Connection

ValetGuidanceSystem was also connected to ParkingTrafficProvider.

A helper method was added:

C#UpdateTrafficProviderData()

It sends traffic data to the ETA system using:

C#parkingTrafficProvider.SetExternalTrafficData(
totalCarsInSimulation,
activeCarsCount
);

Current mapping:

totalCarsInSimulation = allParkingSpots.Count
activeCarsCount = activeSessions.Count

This allows the ETA system to include a traffic buffer based on the number of active valet sessions.

## 11. ParkingTrafficProvider Configuration

ParkingTrafficProvider was initially tested in:

Manual Input

Then prepared for real integration through:

External Traffic Orchestrator

Final intended runtime configuration:

Input Mode: External Traffic Orchestrator

The external values are not meant to be manually edited in the Inspector.
They are updated by ValetGuidanceSystem through SetExternalTrafficData(...).

## 12. Billboard 3D Model and UI Placement

The billboard 3D model was imported separately and positioned in the main scene.

A world-space canvas was created under the billboard:

Billboard_A
└── Billboard_Screen_Canvas
    ├── Background_Image
    ├── ETA_Text
    ├── PickUp_Area_Text
    └── Status_Text

The background image is currently a placeholder and does not require a Figma PNG to test the ETA logic.

## 13. Billboard Text Setup

Three TextMeshPro UI fields were created:

ETA_Text
PickUp_Area_Text
Status_Text

Initial placeholder values:

ETA_Text          → --:--
PickUp_Area_Text  → PickUp -
Status_Text       → Waiting for request

These values are only startup placeholders.
At runtime, they are overwritten by BillboardReceiver when ETA events are received.

## 14. BillboardReceiver Wiring

The BillboardReceiver component must be placed on:

Billboard_Screen_Canvas

and wired as follows:

Time Text        → ETA_Text
Pickup Tag Text  → PickUp_Area_Text
Status Text      → Status_Text

This allows the billboard UI to receive and display ETA updates from SimulationEvents.

## 15. Debug Tester Handling

DynamicRouteDebugTester remains available for manual route testing, but for the real integrated pipeline it should be disabled.

Reason:

The route is now provided by ValetGuidanceSystem, not by manual debug input.

If left active with an empty route list, pressing the debug key R may generate unnecessary warnings.

## 16. Final Runtime Flow

The final integrated flow is:

ValetGuidanceSystem
    selects pickup spot
    calculates List<TrafficNode> path
        ↓
DynamicRouteBridge
    converts TrafficNode route to Transform route
        ↓
CarETAOrchestrator
    calculates ETA:
        base travel time
        + pedestrian crossing delay
        + traffic buffer
        + safety buffer
        ↓
SimulationEvents
        ↓
BillboardReceiver
        ↓
Billboard UI updates

The vehicle still moves through the existing main project logic:

SmartCarNavigator.AssignRouteAndSpot(path, pickupSpot)

The ETA system does not interfere with vehicle movement. It only listens to the same route data and computes/display ETA information.